Information correct as of September 2007

Operating Instructions								Plant/Plant Section:

according to the Occupational Health and Safety Act and

Accident Prevention Regulations

VSG 3.1

# Mowing (lawn mower, overgrowth mower, 
sickle bar mower)

## Dangers to Humans and the Environment

- Foreign objects flying through the air can cause injuries.

- Incision wound on contact with the cutting tool.

- Risk of burns from hot machine parts.

- Exhaust fumes, vibrations, noise, dusts.

- Leaking oils and fuels can endanger the environment.

### Protective measures and rules of conduct

- Observe accident prevention regulation 3.1 and the operating instructions.

- The mowing equipment may only be operated by trained personnel.

- Wear safety shoes and ear protection if necessary when mowing.

- Use a safety filler neck for refuelling and do not smoke!

- When mowing always maintain a sufficient safety distance (refer to the information

provided by the manufacturer !) to other people.

- Foreign objects on the surface to be mowed must be removed beforehand.

- On embankments always mow across the slope. If there is a risk of slipping, the mower

is secured by a second person with a rope, carrying climbing irons if necessary.

- In the case of blockages, first stop the engine and wait until the cutting tool

has come to a standstill.

- The cutting height may only be adjusted when the tools are at a standstill.

- The mower unit must be switched off outside the area being cared for.

- Mowing equipment may only be transported when the engine has been switched off and the cutting tool has come to a standstill.  Ensure that the load is adequately secured!

### What to do in the case of danger or malfunctions

- Switch off the motor and wait for the tool to come to a stop.

- First remove the spark plug connector before working on the cutting tool.

### What to do in the case of accidents, first aid

First aider: Mr/Ms ..........................					Emergency call: 112

- Take immediate action at the scene of the accident.

- Call an ambulance/doctor.

- Inform the business owner/supervisor.

### Maintenance

- Follow the manufacturer's operating instructions for servicing and maintenance.

- Before starting work, the safety and protective devices must be checked and the cutting tool inspected to ensure that it is in a proper condition.

- Repairs must only be performed by specialist personnel.

____________________               _____________________________

Date				     Signature of the business operator

It is hereby confirmed that the contents of these operating instructions conform to the operating conditions and findings of the risk assessment.