|  | Business: |  | Operating Instructions | Date: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Working area: Milking parlour |  | Activity: Milking the cows | Signature: |  |  |
|  | DESIGNATION |  |  |  |  |  |
|  | Working in the milking parlour |  |  |  |  |  |
|  | DANGERS TO HUMANS AND THE ENVIRNOMENT |  |  |  |  |  |
|  |  | - Danger of slipping due to damp and smooth floors. - Danger of falling when climbing the milking parlour stairs. - Being kicked or pushed by the cows. - Danger from electrical installations. - Danger from hazardous substances (detergents/disinfectants). - Danger from a damp workplace. |  |  |  |  |
|  | PROTECTIVE MEASURES AND RULES OF CONDUCT |  |  |  |  |  |
|  |  | - Use anti-slip floor coverings and keep them clean. - Wear safety shoes/boots (S3/S5). - Install anti-slip steps and replace worn treads in good time. In the case of new buildings, plan a ground-level milking parlour entrance if possible. - Deploy only instructed and trained individuals to deal with cattle. - Targeted and careful approach to the cows before touching the udder. Use aids for restless cows (e.g. kickbar). - Keep escape routes clear. - Have the electrical system checked regularly by a specialist. Check the function of the residual current circuit breaker (30 mA) (press the test button). Applies to the entire socket circuits. - When working with hazardous substances, use personal protective equipment in accordance with the manufacturer's safety data sheet. - Wear skin-friendly milking gloves (e.g. made of nitrile). Wear a rubber apron. - Provide and use skin cleansing and skin care products. |  |  |  |  |
|  | WHAT TO DO IN THE CASE OF MALFUNCTIONS |  |  |  |  |  |
|  |  | - Repair defective structural equipment or report a requirement. - In the event of malfunctions in the electrical system, contact a specialist company. |  |  |  |  |
|  | WHAT TO DO IN THE CASE OF ACCIDENTS – FIRST-AID – EMERGENCY TEL. NO. 112 |  |  |  |  |  |
|  |  | - Carry out emergency measures at the scene of the accident! - Call the emergency services/an emergency doctor! First aider:………………………………………………………………………………………... |  |  |  |  |
|  | MAINTENANCE |  |  |  |  |  |
|  |  | - Check the systems and equipment before starting work. - Observe the manufacturer's specifications for maintenance intervals. - Thorough cleaning and care of the equipment after milking has been completed. |  |  |  |  |
|  | CONSEQUENCES OF NON-COMPLIANCE |  |  |  |  |  |
|  |  | - Health consequences: injury, illness! |  |  | www.svlfg.de |  |