Information correct as of September 2007

Operating Instructions								Plant/Plant Section:

according to the Occupational Health and Safety Act and

Accident Prevention Regulations

VSG 3.1

# Hedge Trimmers

## Dangers to Humans and the Environment

- Incision wound on contact with the cutting tool.

- Eye injury caused by twigs and branches.

- Risk of burns from hot machine parts.

- Exhaust fumes, vibrations, noise, dusts, electricity.

- Leaking oils and fuels can endanger the environment.

### Protective Measures and Rules of Conduct

- Comply with the operating instructions.

- Safety shoes, hand and eye protection and, if necessary, ear protection must be worn when

working with hedge trimmers.

- Use a safety filler neck for refuelling and do not smoke!

- Use benzene-reduced fuels.

- Do not use electric hedge trimmers in wet weather. Avoid damage to the cable by cutting

tools or sharp-edged objects.

- Always keep a sufficient safety distance from other people when cutting.

Hedge trimmers must always be operated with both hands.

- Any aids used (e.g. scaffolding) must be stable.

- During transport, the sharp-edged cutting tool must be secured against contact.

- Ensure that the load is adequately secured!

### What to do in the case of danger or malfunctions

- Switch off the motor and wait for the tool to come to a stop.

- Before carrying out maintenance and cleaning work, remove the mains plug or spark plug connector.

### What to do in the case of accidents, first aid

First responder: Mr/Ms ..........................					Emergency call: 112

- Take immediate action at the scene of the accident.

- Call an ambulance/doctor.

- Inform the business owner/supervisor.

### Maintenance

- Follow the manufacturer's operating instructions for servicing and maintenance.

- Before starting work, the safety and protective devices must be checked and the cutting tool inspected to ensure that it is in a proper condition.

- Repairs must only be performed by specialist personnel.

____________________               _____________________________

Date				     Signature of the business operator

It is hereby confirmed that the contents of these operating instructions conform to the operating conditions and findings of the risk assessment.