| Company: |  | Operating instructions according to §§ 4, 9, 12 ArbSchG, § 9 BetrSichV, § 3 VSG 1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Driving silos |  |  |  |  |
| Dangers for humans |  |  |  |  |  |  |
| - Hazard due to vehicle traffic, vehicle crashes - Slipping/sliding and misstepping when getting on and off vehicles - Danger of falling in edge areas when compacting with vehicles - Slipping, tripping and falling on the silo cover or the slippery silo floor - Danger from falling silage masses at the extraction point - Falling from the cutting edge or the driving silo wall - Hazard due to nitrous gases |  |  |  |  |  |  |
| Protective measures and rules of conduct |  |  |  |  |  |  |
| - Prevent unauthorised persons (spectators) from entering the driving silo. - Coordinate work with other parties involved (e.g. subcontractors). - Check protective devices for stability and functionality. - Wear safety shoes/boots (S3/S5). - Fasten seat belts on vehicles. - Only use suitable removal technology with overhead guard. - Do not jump on or off vehicles. Dismount backwards. - Regularly clean the access ladders of vehicles for silage or substrate removal. - Do not overfill the silo (risk of vehicles overturning). - Immediately after filling and compacting, cover the silo airtight and weigh it down sufficiently. - Do not open bulges in the silo film (danger of nitrous gases escaping). - When uncovering the silage film, keep a minimum distance of 2 m from the cutting edge. If necessary, use suitable aids to pull back the film or remove the sandbags/tires, or use mobile railings or rope protection with suitable anchor points. - Avoid undermining at the extraction point or remove immediately. - Secure driving silo walls against persons falling inwards and outwards, e.g. by railings or slopes (vertical distance between slope and upper edge of wall 1 m). |  |  |  |  |  |  |
| Behaviour in the event of disruptions |  |  |  |  |  |  |
| - When recovering vehicles, only select suitable attachment points and use suitable lifting gear. When nitrous gases (brown/rusty red gases) escape from the driving silo: - Immediately cordon off danger areas over a wide area (observe changing wind directions). - Do not enter the danger zone. - Actively ventilate the silo. - Inform superiors . |  |  |  |  |  |  |
| Behaviour in case of accidents, first aid |  |  |  |  | Emergency call 112 |  |
| Location Phone: | First aider: |  |  | Location first aid kit: |  |  |
| - Keep calm, secure the accident site, pay attention to your own safety! - Make an emergency call (112)! - First aid! - Check vital functions and initiate life-saving measures (lateral position, resuscitation, etc.). - In case of threatening bleeding: If possible, hold up the injured body part and stop the bleeding (press a cloth on the wound site, apply a pressure bandage if necessary). |  |  |  |  |  |  |
| Maintenance |  |  |  |  |  |  |
| - Repairs, maintenance work and inspections may only be carried out by persons authorised to do so. - Replace or repair missing or damaged guardrails immediately. |  |  |  |  |  |  |
| Place: Date: |  |  | Signature of responsible person: |  |  |  |
| It is confirmed that the contents of these operating instructions are in accordance with the operational conditions and the findings of the risk assessment. |  |  |  |  |  |  |