|  | Name of the company: |  | Operating Instructions | Date: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Working area: Clamp silage |  | Activity: Filling, driving, removal | Signature: |  |  |
|  | DESIGNATION |  |  |  |  |  |
|  | Clamp silages |  |  |  |  |  |
|  | DANGERS TO HUMANS AND THE ENVIRNOMENT |  |  |  |  |  |
|  |  | - Danger from vehicle traffic, vehicle crash. - Slipping and missing a step when getting on and off vehicles. - Slipping, stumbling and falling on the silo cover or the slippery silo floor. - Danger from falling silage masses at the removal point. - Falling from the cutting edge or the silo wall. x |  |  |  |  |
|  | PROTECTIVE MEASURES AND RULES OF CONDUCT |  |  |  |  |  |
|  |  | x - Prevent unauthorised individuals (onlookers) from entering the silo. - Employees and other involved parties (e.g. service providers) should receive instruction. - Wear safety shoes/boots (S3/S5). - Clean ascents of vehicles for silage or substrate removal regularly! - Do not jump on to or down from vehicles, e.g. for silage or substrate removal. Climb off backwards - Only use appropriate removal technology with a driver protection roof. - Avoid undercuts at the removal location or eliminate them immediately. - When uncovering the silo foil, a minimum distance of 2 metres from the cutting edge must be maintained. If necessary, suitable aids should be used to retract the film or remove the sandbags/tyres, or mobile railings or rope protection with suitable attachment points are used. - Mobile silo walls are secured against persons falling inwards and outwards, e.g. by railings or an embankment (vertical distance between the embankment and top edge of the wall 1 m). - Check protective devices for stability and functionality. x |  |  |  |  |
|  | WHAT TO DO IN THE CASE OF MALFUNCTIONS |  |  |  |  |  |
|  |  | - Immediately cordon off the danger areas! |  |  |  |  |
|  | WHAT TO DO IN THE CASE OF ACCIDENTS – FIRST-AID – EMERGENCY TEL. NO. 112 |  |  |  |  |  |
|  |  | - Carry out emergency measures at the scene of the accident! - Call the emergency services/an emergency doctor! First aider:………………………………………………………………………………………... |  |  |  |  |
|  | Maintenance |  |  |  |  |  |
|  |  | - Replace or repair missing or damaged fall protection devices immediately! |  |  |  |  |
|  | CONSEQUENCES OF NON-COMPLIANCE |  |  |  |  |  |
|  |  | - Health consequences: Possibility of very serious or fatal injuries! x |  |  | www.svlfg.de |  |