Information correct as of September 2007

Operating Instructions								Plant/Plant Section:

according to the Occupational Health and Safety Act and

Accident Prevention Regulations

VSG 1.1, VSG 3.1

# Attachment for Soil Cultivation

# (e.g. mounted rotary tiller, spade machine, lifting ploughs)

## Dangers to Humans and the Environment

- Injuries during attachment and disassembly.

- Crushing between vehicle and attachment.

- Injuries due to overturning of dismantles attachments.

- Injuries caused by foreign objects being ejected or thrown.

- Pulling in and injuries caused by rotating tools or cardan shafts.

- Dangers resulting from being in public road traffic.

- Dangers from buried pipes.

- Injuries caused by remaining in the danger area of the vehicle and the attachment.

- Noise.

- Mineral and organic dusts.

- Recoil.

- Overloading the tractor.

- Injuries from hydraulic line breakage (high-pressure hydraulic oil).

### Protective measures and rules of conduct

- Machines may only be operated by people who have been instructed in their use.

- The corresponding operating instructions must be complied with.

- Do not step between the tractor and attachment during attachment and disassembly.

- Attach, use and secure all safety equipment on the cardan shaft. Comply with the operating

instructions for cardan shafts.

- The attachment must be adjusted so that all working tools are covered or penetrate into the ground.

Protective devices (e.g. baffle plates) are in a protected position during operation.

- No one may remain in the danger zone of the attachment during operation.

- Safety shoes, ear protection and hand protection must be worn if necessary when handling machines and equipment.

- Park attachments only on level ground. Any supporting devices present must be used.

Use supports when working under raised attachments.

- Do not overload the carrier vehicle with attachments. Driving characteristics can be negatively

affected (e.g. steering, braking distance, tilting behaviour, driving off, etc.)

- When driving on the road:

- Excess-wide or extra-long attachments must be marked with warning signs.

- Lighting equipment on the towing vehicle or attachment must not be covered up or dirty.

- Lock hydraulically driven machine parts in place.

### What to do in the case of danger or malfunctions

- Immediately turn off the engine in case of danger.

- Carry out maintenance and cleaning work only when the motor is off and the tool drive is switched off.

### What to do in the case of accidents, first aid

First aider: Mr/Ms ..........................					Emergency call: 112

- Take immediate action at the scene of the accident.

- Call an ambulance/doctor.

- Inform the business owner/supervisor.

### Maintenance

- Carry out a check of the functioning and completeness of the safety and protective devices every time before putting them into operation.

- Repairs may only be carried out by qualified individuals (skilled people).

- Follow the manufacturer's operating instructions for servicing and maintenance.

___________________               _____________________________

Date				     Signature of the business operator

It is hereby confirmed that the contents of these operating instructions conform to the operating conditions and findings of the risk assessment.