# Operating Instructions

in accordance with GefStoffV and VSG 4.5

…

These SAMPLE operating instructions have to be adapted to the operating conditions!

Business:

# Workplace/field of work:                   Cleaning of the milking system

|  | ## Hazardous substance designation |  |  |
| --- | --- | --- | --- |
|  | ##### Nitric acid, phosphoric acid |  |  |
|  | ### Dangers to Humans and the Environment |  |  |
| Social Insurance for Agriculture, Forestry and Horticulture Danger | - Causes burns to the eyes, skin and mucous membranes. - Swallowing causes burns to the stomach and intestines. - The product is slightly hazardous to water. |  |  |
| Keep operating instructions w i t h i n e a s y r e a c h! Protective measures and rules of behaviour |  |  |  |
|  | Further details of the supplier providing the safety data sheet. - Eye protection: Wear tightly fitting safety goggles. - Hand protection: Wear chemical-resistant protective gloves. - Body protection: Wear acid-resistant protective clothing (rubber boots, rubber apron). - Keep containers tightly closed. Ensure good ventilation. - Keep away from food, drinks and animal feed. - Must not enter drains, water bodies. - Never add water. - Never mix with alkaline substances. |  |  |
| What to do in the case of accidents |  |  |  |
|  | - Implement the alarm, escape and rescue plans. - Ensure adequate ventilation. - Avoid contact with the eyes, skin and clothing. - Precipitate gases, vapours and mists with a water spray jet. |  |  |
| First Aid |  |  |  |
|  | - Always ensure self-protection when applying first aid measures. - After contact with the eyes: Rinse for 10 minutes under running water with the eyelids open wide or use an eye rinsing solution. Always consult an ophthalmologist! - After contact with the skin: Remove contaminated clothing immediately and clean the skin. - After inhaling: Ensure the supply of fresh air. If unconscious, keep the airways clear. - After swallowing: Do not induce vomiting. - If you feel unwell, seek medical advice immediately. - First aider: …………….………………… Doctor: ……………………………………………. #### Poison Information Centre: 0228/ 19240 Emergency telephone number: 112 |  |  |
| Proper disposal |  |  |  |

Do not empty into drains or surface waters. In the case of accidental release, absorb with liquid-binding material (e.g. diatomaceous earth, vermiculite or sand). Arrange for disposal in accordance with the details in the safety data sheet (section 13).

Date: ………………201….                  Contractor’s signature: ………………………………………………..

Date: April 2015