# Operating Instructions

acc. to the German Ordinance on Hazardous Substances and VSG 4.5

These EXAMPLE operating instructions must be adapted to the operating conditions.

Business:

# Work area:

Social security Agriculture Forestry and horticulture

Type of work:            Anaesthesia with isoflurane during piglet castration

|  | ## hazard designation |  |  |
| --- | --- | --- | --- |
|  | ##### Isoflurane |  |  |
|  | ### Risks for people and the environment |  |  |
|  | - The main intake route is through the respiratory tract. Inhalation may cause damage to health. - May irritate the respiratory tract. Causes mucous membrane irritation. - May damage the cardiovascular and central nervous systems when inhaled. - The inhalation of high vapour concentrations may cause drowsiness, dizziness or unconsciousness. - Slightly volatile. - Slightly hazardous to water, as heavier than water and hardly miscible with water. - Not flammable. |  | Keep operating instructions in a safe place! |
| Protective measures and rules of conduct |  |  |  |
|  | - Provide good ventilation of the working area (3 to 5 air changes per hour). - May only be stored and transported by trained personnel (expert knowledge) in compliance with the safety regulations. - Wear protective gloves for hygienic reasons. - Do not smoke, eat or drink. - Avoid the inhalation of vapours and skin contact. - Store containers in a well-ventilated location. Do not store together with food or animal feed. Store away from gases (e.g. oxygen). - Comply with employment restrictions for expectant and breastfeeding mothers. |  |  |
| Behaviour in the event of accidents |  |  |  |
|  | - If there is a risk of isoflurane being released, leave the danger area. - To re-enter the scene of the accident, wear a full-face mask with an AX filter, as well as eye, hand and body protection. - Remove wet clothing. - A fire may cause the release of carbon monoxide, carbon dioxide, hydrogen fluoride and hydrogen chloride. Wear self-contained breathing apparatus and a chemical protective suit. |  |  |
| First aid |  |  |  |
|  | - Always ensure self-protection during any first aid measures. - In the case of eye contact, rinse for at least 10 minutes; seek medical attention. - In the case of skin contact, clean the skin with soap under running water. - If swallowed, rinse the mouth and seek medical attention. - After inhalation, remove injured persons from the danger area and ensure the supply of fresh air. - Consult a doctor in the event of symptoms. - Immediate life-saving measures must be taken, depending on the situation. - First responder: ……………...………………… Doctor……………………………………. #### Poison Information Centre: 0228/ 19240 Emergency: 112 |  |  |
| Proper disposal |  |  |  |

Date: ……………… 20….                   Signature of the entrepreneur………………………………………………:  ……

(February 2019)

Not hazardous waste according to the Waste Catalogue Ordinance (AVV). If recovery is not possible, wastes must be disposed of in accordance with local regulations.