|  | Company: |  | Operating Instructions | Date: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Working area: stable complex |  | Activity: Working in the freestall barn | Signature: |  |  |
|  | DESIGNATION |  |  |  |  |  |
|  | Working in the freestall barn |  |  |  |  |  |
|  | DANGERS TO HUMANS AND THE ENVIRNOMENT |  |  |  |  |  |
|  |  | - Danger of slipping due to damp and smooth floors - Being kicked or pushed by the cows - Danger from electrical installations. - Danger from dust, liquid manure gases and animal hair allergens - Risksm from zoonoses |  |  |  |  |
|  | PROTECTIVE MEASURES AND RULES OF CONDUCT |  |  |  |  |  |
|  |  | - Use anti-slip floor coverings and keep them clean. - Wear safety shoes/boots (S3/S5). - Deploy only instructed and trained individuals to deal with cattle. - Provide immobilisation devices such as a grab feeding fence or treatment stand. - Immobilise cattle in immobilisation devices during treatments or examinations. - Stud bulls are to be kept in a separate stud bull box if possible. - Stable complexes with free-running stud bulls are not to be entered without assistants. - Create a personnel pass-through as an escape route from the animal area. - Avoid dust formation and animal hair allergens in the stable (e.g. bind meal-containing feed with liquid, store concentrated feed in the outside silo, if possible attach a cow brush outside the stable). - When stirring liquid manure, open all stable gates and windows wide! - Immediately treat cattle with infectious diseases (e.g. calf lichen). - In stables with slide-type manure removal systems, the run-up points on winches and deflection rollers must be effectively secured against access. - Fans suspended deeper than 2.70 m must be secured against access with a protective grille. - Have the electrical system checked regularly by a specialist. Check the function of the residual current circuit breaker (30 mA) (press the test button). Applies to the entire socket circuits. - Electrically operated roller shutters and curtains must be equipped with an automatic switch-off device or a dead man's switch. |  |  |  |  |
|  | WHAT TO DO IN THE CASE OF MALFUNCTIONS |  |  |  |  |  |
|  |  | - Repair defective structural equipment or report a requirement. - Report any cattle with behavioural problems and remove them from the herd. - In the event of malfunctions in the electrical system, contact a specialist company. |  |  |  |  |
|  | WHAT TO DO IN THE CASE OF ACCIDENTS – FIRST-AID – EMERGENCY TEL. NO. 112 |  |  |  |  |  |
|  |  | - Carry out emergency measures at the scene of the accident! - Call the emergency services/an emergency doctor! First aider:…………………………………………………………………………………... |  |  |  |  |
|  | MAINTENANCE |  |  |  |  |  |
|  |  | - Check the systems and equipment before starting work. - Observe the manufacturer's specifications for maintenance intervals. |  |  |  |  |
|  | CONSEQUENCES OF NON-COMPLIANCE |  |  |  |  |  |
|  |  | - Health consequences: injury, illness! |  |  | www.svlfg.de |  |