|  | Company: |  | Operating Instructions |  | Date: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Working area: |  | Activity: |  | Signature: |  |  |
|  | DESIGNATION |  |  |  |  |  |  |
|  | Brush cutter |  |  |  |  |  |  |
|  | General operating instructions for handling brush cutters |  |  |  |  |  |  |
|  | DANGERS TO HUMANS AND THE ENVIRNOMENT |  |  |  |  |  |  |
|  |  | ▪ Risks incurred by staying in the danger zone! ▪ Dangers associated with rotating, sharp tools (cuts)! ▪ Dangers associated with thrown-off parts! ▪ Dangers due to noise! ▪ Dangers due to exhaust gases! ▪ Dangers associated with hand-body vibrations! |  |  |  |  |  |
|  | PROTECTIVE MEASURES AND RULES OF CONDUCT |  |  |  |  |  |  |
|  |  | Hearing protection: Face protection: Eye protection: Body protection: Foot protection: Hand protection: |  | Wear hearing protection! Wear face protection! Wear safety goggles (in addition to the face protection)! Wear tight-fitting work clothes! Wear safety shoes! Wear protective gloves! |  |  |  |
|  |  | Conduct ▪ The accident prevention regulations and the manufacturer's operating instructions must be complied with! ▪ Brush cutters may only be operated by trained personnel! ▪ Persons under the age of 18 may only operate brush cutters for educational purposes only and under expert supervision! ▪ Individuals under the age of 15 may not work with brush cutters! ▪ When refuelling, use a safety filling nozzle and do not smoke! ▪ Use special low-benzene fuels! ▪ Before starting work: Adjust the carrying strap to the size of your body and check the tools and protective equipment!! ▪ When the device is started it should be firmly supported! ▪ Guide the machine with both hands! ▪ Ensure your footing is secure! ▪ Be careful for foreign bodies when cutting and remove these if they occur! ▪ Adhere to the safety distance (min. 5 m, in the public area min. 15 m)! ▪ The machine must only be transported with the tool protection in place!! ▪ Take into account the tension and pressure conditions in the wood! |  |  |  |  |  |
|  | WHAT TO DO IN THE CASE OF MALFUNCTIONS: |  |  |  |  |  |  |
|  |  | ▪ When dealing with malfunctions, switch off the machine and wait until the tool is stationary! ▪ Always wear protective gloves when working on the tool! |  |  |  |  |  |
|  | WHAT TO DO IN THE CASE OF ACCIDENTS – FIRST-AID – EMERGENCY TEL. NO. 112 |  |  |  |  |  |  |
|  |  | Carry out emergency measures at the scene of the accident! Call an ambulance/doctor! Inform supervisors and the occupational insurance association! First aider: ..............................................……………………….. |  |  |  |  |  |
|  | MAINTENANCE |  |  |  |  |  |  |
|  |  | ▪ Before each use, check the function and safety devices of the machine! ▪ Comply with the manufacturer's guidelines with respect to maintenance and care! ▪ Repairs must only be performed by qualified persons! |  |  |  |  |  |
|  | CONSEQUENCES OF NON-COMPLIANCE |  |  |  |  |  |  |
|  |  | ▪ Health consequences: injuries and illness! ▪ Consequences under industrial law: warning, reprimand! |  |  |  |  |  |