Correct as of November 2007

Operating Instructions								Plant/Plant Section:

according to the Occupational Health and Safety Act and

Accident Prevention Regulations

VSG 1.1, VSG 3.1

# Drive wheel hoe/single-axle tractor with mounted rotary tiller

## Dangers to Humans and the Environment

- Injuries caused by foreign objects being ejected or thrown.

- Mineral and organic dusts.

- Recoil.

- Leg and foot injuries caused by being run over by the device or slipping when reversing.

- Machine can bounce up (foreign bodies, firm layers of soil).

- Rotating chopping devices.

- Risk of burns when touching hot motor parts.

- Exhaust fumes, vibrations, noise, fuels.

- Leaking fuels can endanger the environment.

### Protective Measures and Rules of Conduct

- Machines may only be operated by individuals who have received instruction and who are at least

15 years old.

- The operating instructions of the manufacturer must be observed.

- Wear safety shoes (S 2) and ear protection if necessary.

- To start, the drive must be disengaged and the tool drive switched off.

- Use a safety filler neck for refuelling.

- Do not smoke when refuelling.

- Adjust the guide rails to the operator's requirements. Always guide the device with both hands.

- Watch out for obstacles when reversing.

- Ensure adequate ventilation when working in enclosed areas (greenhouses, foil tunnels).

- Check the function of the danger switch and the milling tool lock for reverse travel.

- Drive across the slope on embankments. If there is a risk of the machine slipping, it must be held by a second person above the machine with the aid of a rod or rope.

- Maintain a sufficient distance from solid foreign bodies/materials (e.g. paved areas).

- 	The tool drive must be switched off during transport, a change of the deployment location and when turning.

### What to do in the case of danger or malfunctions

- In case of danger, press the danger switch immediately.

### What to do in the case of accidents, first aid

First responder: Mr/Ms ..........................					Emergency call: 112

- Take immediate action at the scene of the accident.

- Call an ambulance/doctor.

- Inform the business owner/supervisor.

### Maintenance

- Repairs must only be carried out by an expert (qualified person).

- Follow the manufacturer's operating instructions for servicing and maintenance.

- Carry out a visual and function test and check the function and completeness of the safety and protective devices every time before putting into operation.

- Carry out maintenance and cleaning work only when the motor is off and the tool drive is switched off.

___________________               _____________________________

Date				     Signature of the business owner

It is confirmed that the contents of these operating instructions correspond to the operating conditions and findings of the hazard assessment.