|  | Company: |  | Operating Instructions | Date: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Working area: |  | Activity: | Signature: |  |  |
|  | DESIGNATION |  |  |  |  |  |
|  | Electric hand tools |  |  |  |  |  |
|  | General operating instructions for handling electric hand tools |  |  |  |  |  |
|  | DANGERS TO HUMANS AND THE ENVIRNOMENT |  |  |  |  |  |
|  |  | ▪ Dangers from electric shock! ▪ Dangers from tools or work pieces that get out of control! ▪ Dangers from catching clothing and hair! ▪ Dangers due to noise! ▪ Dangers from hand-arm vibrations! |  |  |  |  |
|  | PROTECTIVE MEASURES AND RULES OF CONDUCT |  |  |  |  |  |
|  |  | Hearing protection: Eye protection: Body protection: Foot protection: | Wear ear protection when working at noise levels above 80dB(A)! Wear safety goggles! Wear suitable tight working clothes Use safety shoes! |  |  |  |
|  |  | Conduct: ▪ The accident prevention regulations and the manufacturer's operating instructions must be complied with! ▪ When changing tools: Remove the mains plug! ▪ Use only approved tools in electric hand tools! ▪ Use tools only for their intended purpose! ▪ Check connecting cables for damage before switching on! ▪ Check that the safety devices are complete and functioning correctly before use! ▪ Do not dismantle or block protective devices! ▪ Guide electric hand tools with both hands and make sure that your foothold is stable! ▪ Use electrical hand tools only together with a mains supply protected by an RCD switch (30mA)! ▪ If you have long hair: Use a hairnet! ▪ Do not wear gloves when using drilling machines! |  |  |  |  |
|  | WHAT TO DO IN THE CASE OF MALFUNCTIONS |  |  |  |  |  |
|  |  | ▪ Replace defective tools immediately! ▪ Have damage to electrical cables and components repaired by a specialist! |  |  |  |  |
|  | WHAT TO DO IN THE CASE OF ACCIDENTS – FIRST-AID – EMERGENCY TEL. NO. 112 |  |  |  |  |  |
|  |  | Carry out emergency measures at the scene of the accident! Call an ambulance/doctor! Inform supervisors and the occupational insurance association! First aider: ..............................................……………………….. |  |  |  |  |
|  | MAINTENANCE |  |  |  |  |  |
|  |  | ▪ Before each use, check the function and safety devices of the machine! ▪ Comply with the manufacturer's guidelines with respect to maintenance and care! ▪ Repairs must only be performed by qualified persons! ▪ Electric hand-held devices must be checked annually by a qualified electrician to ensure that they are in proper condition (keep test logbook).! |  |  |  |  |
|  | CONSEQUENCES OF NON-COMPLIANCE |  |  |  |  |  |
|  |  | ▪ Health consequences: injuries and illness! ▪ Consequences under industrial law: warning, reprimand! |  |  |  |  |