|  | Firm: |  | Operating Instructions | Date: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Work area: |  | Activity: | Signature: |  |  |
|  | DESIGNATION |  |  |  |  |  |
|  | Compact Loader |  |  |  |  |  |
|  | Loading and Transport Work |  |  |  |  |  |
|  | DANGERS TO PEOPLE AND THE ENVIRONMENT |  |  |  |  |  |
|  |  | ▪ Danger of being run over ▪ Danger due to slipping when getting on and off ▪ Danger from turning over ▪ Danger from hydraulic oil leaking under high pressure ▪ Danger from falling objects (in particular large bales) ▪ Danger from remaining in the danger area |  |  |  |  |
|  | PROTECTIVE MEASURES AND RULES OF CONDUCT |  |  |  |  |  |
|  |  | Foot protection: Hearing protection: | Wear safety shoes! Wear hearing protection at more than 80 dB(A)! |  |  |  |
|  |  | Behaviour: ▪ Only to be operated by reliable individuals who have received instruction and been asked to operate the device by the company ▪ The accident prevention regulations and the manufacturer‘s operating instructions must be complied with! ▪ To be checked before use: Service and parking brake, working tools, steering, hydraulics, lifting device, lighting and warning device. ▪ During use: Check the condition and load-bearing capacity of the road surface – transport load in the lowest position, be aware of the increased risk of tilting with full steering lock and a raised load – therefore drive at an appropriate speed. ▪ Parking the vehicle: Lower the lifting device, operate the parking brake, remove the key. ▪ Do not remain in the danger area. ▪ Riding on the machine or working equipment is prohibited! ▪ Use a banksman if the visibility is not good. ▪ For stacking and unstacking large bales use only vehicles with a cabin or protective roof for the driver. ▪ Move folding protective devices to the protective position. ▪ Use the driver restraint system during all operations. ▪ Don´t smoke when refuelling! |  |  |  |  |
|  | WHAT TO DO IN THE CASE OF MALFUNCTIONS |  |  |  |  |  |
|  |  | Fire: Malfunction: | ▪ Fight incipient fires with a fire extinguisher – call the fire brigade! ▪ In the case of danger, stop the engine immediately! ▪ For maintenance, cleaning and repair work: Remove the ignition key and wait until the engine has come to a standstill, then lower the load! ▪ Continue working only after the malfunction has been repaired by trained personnel! |  |  |  |
|  | WHAT TO DO IN THE CASE OF ACCIDENTS - FIRST AID - EMERGENCY CALL 112 |  |  |  |  |  |
|  |  | ▪ Carry out emergency measures at the scene of the accident! ▪ Call an ambulance/doctor! ▪ Inform superiors and professional association! First aider: ………………………………………………………………………………………….. |  |  |  |  |
|  | MAINTENANCE |  |  |  |  |  |
|  |  | ▪ Always check the function and safety equipment of the machine before switching on! ▪ Comply with the manufacturer’s instuctions regarding care and maintenance! ▪ Have repairs carried out only by trained personnel! |  |  |  |  |
|  | CONSEQUENCES OF NON-COMPLIANCE |  |  |  |  |  |
|  |  | ▪ Consequences for health: Injuries and illness! ▪ Consequences under employment law: Warning, reprimand! |  |  |  |  |